CO2_readme.txt

University of New Hampshire Coastal Ocean Observing Center

Underway Carbon Dioxide Data description

Investigators: Doug Vandemark, Joe Salisbury, Chris Hunt, Wade McGillis

Contact: Doug Vandemark (doug.vandemark@unh.edu)

PLEASE NOTE: Users of data will be required to provide proper credit and acknowledgment of the provider.  Users of data are encouraged to discuss relevant findings with the provider early in the research.  The user is required to give all providers of the data being used a copy of any manuscript resulting from use of the data prior to the initial submission for publication, thus giving the data provider an opportunity to comment on the paper. The provider(s) shall have the right to be named as a co-author.

List of variables included in this dataset:

COLUMN	HEADER		EXPLANATION

=======	=======		============

1.		year			Year measurements were taken

2.		day_of_year		Sequential day of year measurements were taken

3.		secDay			Sequential seconds of day of measurement (e.g. 60,000

secDay=16:40:00)

4.		lat			Latitude in decimal degrees (negative values are in the

Southern Hemisphere)

5.		lon			Longitude in decimal degrees (negative values are in the

Western Hemisphere)

6.		speed			Ship speed in meters per second

7.		Water_temp		Sea surface temperature measured by a SBE-45

thermosalinograph, in degrees Celcius

8.		salinity			Sea surface salinity measured by a SBE-45

thermosalinograph, on the Practical Salinity Scale

9.		pressure_atm		Barometric pressure from either ship’s barometer, nearby

buoy barometer, or standard atmospheric pressure (1013.25 hPa), in hectopascals

10.		xCO2_w		Mole fraction of CO2 (dry) in equilibrator headspace at

equilibrator temperature, in parts per million

11.		fCO2_w			Fugacity of CO2 in seawater, in microatmospheres

12.		xCO2_a			Mole fraction of CO2 (dry) in air, in parts per million

13.		fCO2_a			Fugacity of CO2 in air, in microatmospheres

14.		air_temp			Air temperature in degrees Celcius

The following fields have been QC’ed by the UNH Coastal Ocean Observing Center:

year

day_of_year

secDay

lat

lon

speed

Water_temp

Salinity

xCO2_w

fCO2_w

xCO2_a

fCO2_a

The following fields came at least partly from National Buoy Data Center data, and cannot be verified:

pressure_atm

air_temp

CO2 Analyzer: Licor 6262 or Licor 840 infrared (IR) analyzer (Li-cor Biosciences Inc, Lincoln NE).

Drying method:  Nafion selectively permeable membrane drying loop (Perma Pure, Toms River NJ) with counter-flowing nitrogen stream.

Equilibrator type:  Three-chamber fast-rate equilibrator similar to that of Wanninkhof and Thoning (1993, reference below), flow rate 1.5 l/min.

CO2 calibration standards:  Two-point calibration using ultrapure nitrogen and a mixture of CO2 in nitrogen, ranging from 832.3 to 856.1 ppm.  Mixtures of CO2 in nitrogen, analyzed to within +/- 1%, were obtained from Scott-Marrin Inc (Riverside, CA).  These standards are then adjusted based on comparison to a series of standards ranging from 378-515 ppmv obtained from NOAA/ESRL (http://www.esrl.noaa.gov/).

Seawater temperature and salinity were measured using a SBE 45 thermosalinograph, pumped at 1.5 l/min.  Tests using a handheld temperature and salinity sensor (YSI 85, YSI Instruments Yellow Springs OH), aboard the RV Gulf Challenger indicate temperature differences are less than 0.01 degree Celcius between equilibrator outflow and intake sea surface water.  Therefore, no temperature correction is applied to data taken aboard the RV Gulf Challenger.  A temperature correction of 0.15 degrees Celcius is applied to data taken aboard the RV Oceanus (cruises from 10/31/2006 to 11/11/2006).

CO2 data calculation: Corrections of the data for water vapor pressure and sea surface temperature and conversion from pCO2 to the fugacity of carbon dioxide (fCO2) were carried out according to the DOE handbook (1994).

CO2 data quality: all data, excepting those from 20040721, have been verified to an accuracy of +/- 2 mmol/mol or uatm.

References:

DOE (2007) Guide to Best Practices for Ocean CO2 Measurements, A.G. Dickson, C.L. Sabine and J.R. Christian, eds.  PICES Special Publication, 191 pp.

Wanninkhof, R. and Thoning, K.  1993.  Measurement of fugacity of CO2 in surface water using continuous and discrete sampling methods.  Marine Chemistry 44: 189-204.