R/V Ronald H. Brown METADATA - 1997

Class of Data:  Surface ocean and atmospheric carbon dioxide concentrations

Dataset Identifier:  R/V Ronald H. Brown

One File:  RHB1997

Statement of how to cite dataset:

Ron Brown website: http://www.aoml.noaa.gov/ocd/gcc/rvbrown_data1997.php

These data are made freely available to the public and the scientific community

in the belief that their wide dissemination will lead to greater understanding

and new scientific insights.  The availability of these data does not

constitute publication of the data.  We rely on the ethics and integrity of

the user to assure that AOML receives fair credit for our work.  Please send

manuscripts using this data to AOML for review before they are submitted

for publication so we can insure that the quality and limitations of the data

are accurately represented.

Measurement platform identifier:  NOAA research vessel Ronald H. Brown (R104)

Cruise Information:

The Ron Brown was commissioned in July, 1997 and conducted 2 research cruises in

the eastern Pacific.  Also included is data from the transit from the Pacific

to the ship’s home port of Charleston, S.C.

Project Information:

The system was permanently installed on the Ron Brown before the ship was

commissioned and has in most cases been operated by ship’s personnel.  The

work was sponsored by the Underway pCO2 on Ships project of the NOAA climate

program.

Scientist responsible for technical quality of dataset:

Rik Wanninkhof

NOAA/AOML/Ocean Chemistry Division

4301 Rickenbacker Causeway

Miami, Florida 33149

Rik.Wanninkhof@noaa.gov

Contact person for this dataset:

Bob Castle

NOAA/AOML/Ocean Chemistry Division

4301 Rickenbacker Causeway

Miami, Florida 33149

Robert.Castle@noaa.gov

Timestamp for initial submission of dataset:  02/15/08

Timestamp for the most recent update of dataset:  02/18/08

Timestamp period the dataset refers to: 7/28/1997 – 11/14/1997

Geographic area the dataset refers to:

6 N to 47 N

131 W to 79 W

1997 Cruises:

RB199701 –	PACS Leg 1

Panama Canal to San Diego, CA

July 28, 1997 to August 28, 1997

Chief Scientist – Sandra Yuter

Operator – Bob Castle

RB199702 -	PACS Leg 2

San Diego, CA to San Diego, CA

August 29, 1997 to September 6, 1997

Chief Scientist – Sandra Yuter

Operator – Bob Castle

RB199703 -	VENTS Leg 1

San Diego, CA to Newport, OR

September 12, 1997 to September 16, 1997

Chief Scientist – Christopher Fox

Operator – Marilyn Roberts

RB1997094 -	VENTS Leg 2

Newport, OR to Newport, OR

September 20, 1997 to October 5, 1997

Chief Scientist – Christopher Fox

Operator – Dana Greeley

RB199705 -	VENTS Leg 3

Newport, OR to San Francisco, CA

October 11, 1997 to October 25, 1997

Chief Scientist – Gary Massoth

Operator – Marilyn Roberts

RB199706 -	Transit Leg 1

San Francisco, CA to San Diego, CA

October 29, 1997 to October 31, 1997

Chief Scientist – None

Operator – Jonathan Shannahoff

RB199707 -	Transit Leg 2

San Diego, CA to Panama Canal

November 1, 1997 to November 10, 1997

Chief Scientist – None

Operator – Jonathan Shannahoff

RB199707 -	Transit Leg 3

Panama Canal to Key West, FL

November 11, 1997 to November 14, 1997

Chief Scientist – None

Operator – Jonathan Shannahoff

List of variables included in this dataset:

COLUMN	HEADER			EXPLANATION

1.		GROUP/SHIP:			AOML_Brown for all underway data from the Ron Brown.

2.		CRUISE_DESIGNATION:	Cruise ID (e.g., RBYYYYnn where RB = Ron Brown, YYYY

= the four digit year, and nn = the cruise number for

that year).

3.		JD_GMT:			Decimal year day.

4.		DATE_DDMMYYYY:		GMT date.  The date format has been changed to comply

with the IOCCP recommendations.

5.		TIME_HH:MM:SS:		GMT time.

6.		LAT_DEC_DEGREE:		Latitude in decimal degrees (negative values are in 							the southern hemisphere).

7.		LONG_DEC_DEGREE:		Longitude in decimal degrees (negative values are in 							the western hemisphere).

8.		xCO2W_PPM:			Mole fraction of CO2 (dry) in the equilibrator

headspace at equilibrator temperature (Teq) in parts

per million.

9.		xCO2A_PPM:			Mole fraction of CO2 in air in parts per million.

10.		EqTEMP_C:			Temperature in equilibrator water in degrees

centigade.  Temperature in equilibrator measured with

a calibrated thermistor.

11.		PRES_EQUIL_hPa:		Barometric pressure in the lab in hectopascals (1

hectopascal = 1 millibar).

12.		SST(TSG)_C:			Temperature from the ship's thermosalinograph in

degrees centigrade.

13.		SAL(TSG)_PERMIL:		Salinity from the ship's thermosalinograph on the

Practical Salinity Scale.

14.		fCO2w,eq:			Fugacity of CO2 in the equilibrator in

microatmospheres calculated as outlined below.

15.		fCO2W@SST_uatm:		Fugacity of CO2 in sea water in microatmospheres

calculated as outlined below.

16.		fCO2A_uATM:			Fugacity of CO2 in air in microatmospheres

calculated as outlined below.

17.		dfCO2_uatm:			Sea water fCO2 - air fCO2 in microatmospheres.  This

uses the average air value for the current hour.

The following fields have been QC'ed by the CO2 group:

GROUP/SHIP

CRUISE_DESIGNATION

JD_GMT

DATE_DDMMYYYY

TIME_HH:MM:SS

LAT_DEC_DEGREE

LONG_DEC_DEGREE

xCO2W_PPM

xCO2A_PPM

EqTEMP_C

PRES_EQUIL_hPa

fCO2w,eq

fCO2W@SST_uatm

fCO2A_uATM

dfCO2_uatm

The following fields are from the ship's onboard systems and the quality of this

data cannot be verified:

SST(TSG)_C

Sal(TSG)_Permil

Narrative description of system design:

CO2 ANALYTICAL SYSTEM:

The concentration of carbon dioxide (CO2) in surface ocean water is determined

by measuring the concentration of CO2 in gas that is in contact with the water.

Surface water is pumped ~ 100 m through 7/8" Teflon tubing from an inlet

in the ship's bow to the equilibration chamber.  Water comes from the bow

intake ~4.2 m below the water line and the TSG is located close to the inlet.

When the SST is below about 20 oC, friction in the pipes and from the pump cause

heating and the Teq is higher than SST.  When the SST is higher than about 25 oC,

the ship’s air conditioning cools the water and the Teq is lower than SST.

The equilibration chamber has an enclosed volume of gas, or headspace, and a pool

of seawater that continuously overflows to a drain.  As the water flows through the

chamber, the dissolved gases (like CO2) partition between the water and the

headspace.  At equilibrium, the ratio of CO2 in the water and in the headspace is

influenced most by temperature, and that relationship is known.  By measuring

the concentration of CO2 in the headspace and the temperature in the chamber,

the partial pressure (or fugacity) of CO2 in the surface water can be calculated.

INSTRUMENT DESCRIPTION

The general principle of instrumental design can be found in Wanninkhof and Thoning

(1993), Ho et al. (1995), and Feely et al. (1998).  The concentration of CO2 in the

headspace gas is measured using the adsorption of infrared (IR) radiation, which

results from changes in the rotational and vibrational energy state of the CO2

molecule.  The LI-COR detector passes IR radiation through two 6" cells.  The

reference cell is flushed with a gas of known CO2 concentration.  The sample cell

is flushed with the headspace gas.  A vacuum-sealed, heated filament is the

broadband IR source.  The IR radiation alternates between the two cells via a

chopping shutter disc. An optical filter selects an adsorption band specific

for CO2 (4.26 micron) to reach the detector.  The solid state (lead selenide)

detector is kept at -12 degrees °C for excellent stability and low signal

noise (less than 0.2 ppm).

Several steps are taken to reduce interferences and to increase the accuracy

of the measurements.  After the equilibration chamber, the headspace travels

through a drying trap to remove water vapor.  During each analysis, the

headspace gas is compared to a reference gas of known concentration.  To

improve the accuracy of the measurements, three different gaseous standards

for CO2 are analyzed once an hour instead of the headspace gas.

Analyzer:  LI-COR 6251 (analog output) infrared (IR) analyzer.

Method of Analysis:  Differential analyses relative to the low standard.  Measures

dried equilibrator headspace gas.  Gas flow is stopped prior to IR readings.

Drying Method:  The equilibrator headspace sample gas first goes through a glass

condenser cooled to ~ 5 oC.  The sample and standard gases pass through a short

column of magnesium perchlorate before reaching the analyzer.

Equilibrator (setup, size, flows):  The equilibrator is based on a design by R.

Weiss and was fabricated from a plexiglass housing with ~8 L water reservoir and

~16 L gaseous headspace.  Water flow rate is ~11 L/min.  Headspace recirculation

rate is ~200 ml/min.

Additional sensors:

Thermistor mounted in the bottom of the equilibrator.

Setra Barometer Model 370

YSI Model 600R thermosalinograph with temperature, salinity, and dissolved

oxygen sensors.  This TSG is mounted in the Hydro lab sink near the

equilibrator and the two are teed off the uncontaminated seawater feed.

The dissolved oxygen measurements are not reported in the final data file.

Narrative statement identifying measurement method for each required parameter:

CALCULATIONS:

The mixing ratios of ambient air and equilibrated headspace air are calculated

by fitting a second-order polynomial through the hourly averaged millivolt

response of the detector versus mixing ratios of the standards.  Mixing ratios

of dried equilibrated headspace and air are converted to fugacity of CO2 in

surface seawater and water saturated air in order to determine the fCO2.

For ambient air and equilibrator headspace, the fCO2a (or fCO2eq) is calculated

assuming 100% water vapor content:

fCO2eq = xCO2eq(P-pH2O)exp(B11+2*d12)P/RT

where fCO2eq is the fugacity in the equilibrator, pH2O is the water vapor

pressure at the sea surface temperature, P is the atmospheric pressure (in atm),

T is the SST or equilibrator temperature (in K) and R is the ideal gas constant

(82.057 cm^3·atm·deg^-1·mol^-1).  The exponential term is the fugacity correction

where B11 is the second virial coefficient of pure CO2

B11 = -1636.75 + 12.0408T - 0.032795T^2 + 3.16528E-5 T^3

and d12 = 57.7 - 0.118 T is the correction for an air-CO2 mixture in units of

cm^3·mol^-1 (Weiss, 1974).

The calculation for the fugacity at SST involves a temperature correction term

for the increase of fCO2 due to heating of the water from passing through the

pump and through 5 cm ID PVC tubing within the ship.  The empirical temperature

correction from equilibrator temperature to SST is:

fCO2(SST) = fCO2(eq) /

Exp ((Teq-SST) * [0.03107 – 2.7851E-4 * Teq – 1.8391E-3 * ln(fco2eq * 1.0E-6)])

where SST is sea surface temperature and Teq is the equilibrator temperature in

degrees °C.

Sampling Cycle:

The system runs on an hourly cycle during which 3 standard gases, 3 air

samples from the bow tower and 8 surface water samples (from the

equilibrator head space) are analyzed on the following schedule:

Mins. after hour		Sample

4 				Low Standard

8 				Mid Standard

12 				High Standard

16.5				Water

21 				Water

25.5				Water

30				Water

34				Air

38				Air

42				Air

46.5				Water

51				Water

55.5				Water

60				Water

NOTES ON DATA:

Columns have a default value of –999.99 in case of instrument malfunction,

erroneous readings or missing data.  Furthermore, if a suspicious xCO2 value,

pressure or temperature value is encountered, the fCO2 is not calculated.

Analytical Instrument Manufacturer/Model:

The Ron Brown system (version 2.5) was an in-house prototype built by Jason

Masters, Mike Shoemaker, and Bob Castle in 1997.  The analyzer is a LI-COR 6251

(analog output) infrared analyzer.

Standard Gases and Reference Gas:  The three standard gases came from CMDL

in Boulder and are directly traceable to the WMO scale.  While individual data

points above the high standard gas concentration or below the low standard gas

concentration may not be accurate, the general trends should be indicative of the

seawater chemistry.

Description of any additional environmental control:

The system is located in the Hydro Lab of the Ron Brown.  The room is

air-conditioned with little temperature fluctuation.

Resolution of measurement:

The resolution of the instrument is better than 0.1 ppm.

Estimated overall uncertainty of measurement:

The xCO2eq measurements are believed accurate to 0.1 ppm.  The

fCO2@SST measurements are believed to be precise to 0.2 ppm.

List of calibration gases used:

The standards used on all 1997 cruises are:

STANDARD	TANK #	CONCENTRATION	VENDOR

STD1		N/A		300.45		CMDL

STD2		N/A		343.11		CMDL

STD3		N/A		431.31		CMDL

Traceability to an internationally recognized scale (including date/place

of last calibration made):

All standards are obtained from NOAA/CMDL, now called the Global

Monitoring Division of the Earth Research Laboratory and are directly

traceable to WHO scale.

Uncertainty of assigned value of each calibration gas:

The uncertainty based on pre and post cruise calibrations is less

than 0.05 ppm.

Pressure/Temperature/Salinity:

For information about the ship’s thermosalinograph, contact Chief Survey Tech

Jonathan Shannahoff at jonathan.shannahoff@noaa.gov.

Units:

All xCO2 values are reported in parts per million (ppm) and fCO2 values

are reported in microatmospheres (uatm) assuming 100% humidity at the

equilibrator temperature.

Bibliography:

DOE (1994). Handbook of methods for the analysis of the various parameters of the

carbon dioxide system in sea water; version 2. DOE.

Feely, R. A., R. Wanninkhof, H. B. Milburn, C. E. Cosca, M. Stapp and P. P. Murphy

(1998). A new automated underway system for making high precision pCO2

measurements onboard research ships. Analytica Chim. Acta 377: 185-191.

Ho, D. T., R. Wanninkhof, J. Masters, R. A. Feely and C. E. Cosca (1997).

Measurement of underway fCO2 in the Eastern Equatorial Pacific on NOAA

ships BALDRIGE and DISCOVERER, NOAA data report ERL AOML-30, 52 pp.,

NTIS Springfield.

Wanninkhof, R. and K. Thoning (1993). Measurement of fugacity of CO2 in surface

water using continuous and discrete sampling methods. Mar. Chem. 44(2-4):

189-205.

Weiss, R. F. (1970). The solubility of nitrogen, oxygen and argon in water and

seawater. Deep-Sea Research 17: 721-735.

Weiss, R. F. (1974). Carbon dioxide in water and seawater: the solubility of

a non-ideal gas. Mar. Chem. 2: 203-215.

Weiss, R. F., R. A. Jahnke and C. D. Keeling (1982). Seasonal effects of

temperature and salinity on the partial pressure of CO2 in seawater.

Nature 300: 511-513.

Comments related to all 1997 data:

1.	xCO2 values outside the range of the standard gases (i.e. below 300.45 ppm or above 	431.31 ppm) are not as accurate as values within the range.  However, the general 	trends should be indicative of the seawater chemistry.

2.	Data from the ship’s computer system (SST and salinity) were merged into the

underway system’s data file with an approximate 3 minute delay factor.  This is to

account for the time it takes for sea water to travel from the bow intake to the

equilibrator in the Hydro Lab.

3.	The Valco valve that routes the various gases to the LI-COR moved to the wrong

position about 1.5% of the time causing the wrong gas to be passed through the

analyzer.  When this occurred during a headspace or atmospheric gas measurement,

the value was marked bad and removed from the final data file.  When this occurred

during a standard gas phase, the voltage response of the standard gas was

interpolated using the values from the preceding and following hours.

4.	The equation used to convert the equilibrator thermistor voltage to temperature was

incorrect, resulting in values that were approximately 0.9 degrees too high.  In

addition, relay closures affected the voltage readings from the thermistor.  To

obtain correct equilibrator temperature readings, we assumed that the YSI TSG

temperature was the same as the equilibrator temperature since both sample sea

water from the same source in the Hydro Lab.  However, the YSI TSG temperature

readings are known from calibration data to be about 0.2 degrees high.  We

therefore determined a corrected YSI temperature by using calibration data to fit a

linear equation relating observed YSI temperature to actual temperature as given by

a Guildline platinum resistance thermometer (New YSI T = 1.0026885 * Old YSI T –

0.272854). A new equation relating thermistor voltage to the corrected YSI T was

then determined using all the data from leg 2 of the PACS 1997 cruise (T =

6.0955*V^3 -46.862*V^2 + 101.16V - 28.638.

Comments related to the individual legs:

RB199701:	1. The system was shut down 3 times – for 45 minutes on 8/5/97, for 2

hours on 8/18/97, and for 2 hours on 8/20/97.

2. The seawater intake on the Ron Brown was switched from 4.2 to 2.2

meters on 8/9/97 at 1500 and switched back to 4.2 m on 8/23/97 at 2230.

3. After spending most of the cruise on station at 7.8 N, 125 W, the

ship was forced to go to San Diego for a medical evacuation operation.

The system was stopped for a short time and when data collection

resumed, leg 3 began.

RB199702:	1. The first ~ 27 hours of data were removed due to a clogged valve.

The good data begins on August 31.

RB199703:	1. Misalignment of the Valco valve resulted in low gas flows in the

standard 3, standard 1, and air phases.  This problem was especially

bad during the last 2 days of the cruise.  To correct them, I computed

the average LI-COR voltage in both the standard 1 and standard 3 phases

when gas flow exceeded 30 ml/min.  Whenever possible, I interpolated

bad values in these phases using the previous and next good values in

the appropriate phase.  However, when more than 2 consecutive values

were bad, I replaced the bad values with the computed average value.

In several air phases, I deleted the first air value of the group due

to incomplete flushing of the LI-COR sample cell.

RB199704:	1. There was no data from the ship’s computer system for the first two

hours of the leg, so I computed SST and salinity for that time period

from the equilibrator temperature and YSI salinity respectively.  The

YSI salinity data contained some sharp drops which I believe were due

to low flow through the YSI cell.  I removed the first few of these

"dropouts" and interpolated the missing values.  Then I did a linear

curve fit on the first 11 days of good salinity data and on all the

temperature data to derive equations relating YSI salinity to SCS

salinity and equilibrator temperature to SCS temperature.  I used these

computed values for the first 2 hours of missing SCS data.  The

equation for SCS temperature was:  SCS T = 1.236951 * Eq T - 4.612866

with R^2 = 0.996192.  The equation for salinity was SCS S = 0.990094 *

YSI S + 0.385682 with R^2 = 0.984214.

RB199705:	1. There was a high differential between sea surface temperature and

equilibrator temperature that ranged from 0.5 degrees at an SST of 16.5

to 2 degrees at an SST of 12.5.  I attribute this to the relatively low

sea surface temperature that resulted in a greater degree of warming

while the water was transported from the bow intake to the Hydro lab

near the ship's stern.

RB199706:	1. For the first day of this leg, gas flow in the water phase was low.

This may affect some of the water xCO2 values slightly, but it should

be limited to the first measurement in each group of four water

readings.

RB199707:	1. We received no data from the ship's computer system (SCS) for the

first 10 hours of this leg.  For this time period we computed sea

surface temperature and salinity from equilibrator temperature and the

YSI TSG salinity sensor respectively.  The equations used were:  SCS

temperature = (1.02205 * Equilibrator temperature) - 0.44594 and SCS

salinity = (1.00754 * YSI salinity) - 0.16921.

RB199708:	None.