R/V Ronald H. Brown METADATA - 2002

Class of Data:  Surface ocean and atmospheric carbon dioxide concentrations

Dataset Identifier:  R/V Ronald H. Brown

One File:  RHB2002

Statement of how to cite dataset:

Ron Brown website: http://www.aoml.noaa.gov/ocd/gcc/rvbrown_data2002.php

These data are made freely available to the public and the scientific community

in the belief that their wide dissemination will lead to greater understanding

and new scientific insights.  The availability of these data does not

constitute publication of the data.  We rely on the ethics and integrity of

the user to assure that AOML receives fair credit for our work.  Please send

manuscripts using this data to AOML for review before they are submitted

for publication so we can insure that the quality and limitations of the data

are accurately represented.

Measurement platform identifier:  NOAA research vessel Ronald H. Brown (R104)

Cruise Information:

The Ron Brown conducted 9 major cruises in the Atlantic and eastern Pacific

Oceans for a total of 13 legs.

Project Information:

The system was operated by personnel from AOML or PMEL (Pacific Marine 	Environmental Laboratory) or by the Ron Brown’s Chief Survey Tech, Jonathan 	Shannahoff.  The work was sponsored by the Underway pCO2 on Ships project of the 	NOAA climate program.

Scientist responsible for technical quality of dataset:

Rik Wanninkhof

NOAA/AOML/Ocean Chemistry Division

4301 Rickenbacker Causeway

Miami, Florida 33149

Rik.Wanninkhof@noaa.gov

Contact person for this dataset:

Bob Castle

NOAA/AOML/Ocean Chemistry Division

4301 Rickenbacker Causeway

Miami, Florida 33149

Robert.Castle@noaa.gov

Timestamp for initial submission of dataset:  11/18/09

Timestamp for the most recent update of dataset:  11/18/09

Timestamp period the dataset refers to: 1/15/2002 – 11/8/2002

Geographic area the dataset refers to:

15 S to 45 N

115 W to 30 W

2002 Cruises:

RB200201T –	Transit 2002A

Charleston, SC to Recife, Brazil

January 15, 2002 to January 27, 2002

Chief Scientist – N/A

Operator – Jonathan Shannahoff

RB200201 -	Tropical-Subtropical Interaction

Recife, Brazil to Bridgetown, Barbados

February 1, 2002 to February 23, 2002

Chief Scientist – Bob Molinari

Operator – Jonathan Shannahoff

RB200202 -	Plueddemann Moorings

Bridgetown, Barbados to Bridgetown, Barbados

March 3, 2002 to March 7, 2002

Chief Scientist – Albert J. Plueddemann

Operator – Jonathan Shannahoff

RB200203 -	Grenada Passage

Bridgetown, Barbados to St. Thomas, U.S. Virgin Islands

March 10, 2002 to March 18, 2002

Chief Scientist – Albert J. Plueddemann

Operator – Jonathan Shannahoff

RB200204T -	Transit 2002B

St. Thomas, U.S. Virgin Islands to Jacksonville, FL

March 25, 2002 to March 26, 2002

Chief Scientist – N/A

Operator – Jonathan Shannahoff

RB200204 -	Western Boundary Time Series

Jacksonville, FL, to Miami, FL

June 17, 2002 to June 29, 2002

Chief Scientist – Sonia Bauer

Operator – Jonathan Shannahoff

RB200205A -	Air Quality in Coastal New England Leg A

Charleston, SC to Portsmouth, NH

July 13, 2002 to July 26, 2002

Chief Scientist – Tim Bates

Operator – Jonathan Shannahoff

RB200205B -	Air Quality in Coastal New England Leg B

Portsmouth, NH to Charleston, SC

July 26, 2002 to August 11, 2002

Chief Scientist – Tim Bates

Operator – Jonathan Shannahoff

RB200206 -	South Florida Plume Study

Charleston, SC to Miami, FL

August 17, 2002 to August 23, 2002

Chief Scientist – Jules Craynock

Operator – Jonathan Shannahoff

RB200207 -	Ocean Exploration Hudson Canyon

Miami, FL to New York, NY

August 27, 2002 to September 15, 2002

Chief Scientist – Peter Rona

Operator – Jonathan Shannahoff

RB200208T -	Transit 2002C

New York, NY to San Juan, Puerto Rico

September 19, 2002 to September 24, 2002

Chief Scientist – N/A

Operator – Jonathan Shannahoff

RB200208 -	Ocean Exploration Puerto Rican Trench

San Juan, Puerto Rico to San Juan, Puerto Rico

September 24, 2002 to September 30, 2002

Chief Scientist – Uri Ten Brick

Operator – Jonathan Shannahoff

RB200209 -	TOGA/TAO 2002

Balboa, Panama to Balboa, Panama

October 8, 2002 to November 8, 2002

Chief Scientist – Patrick A’Hearn

Operator – Jonathan Shannahoff

List of variables included in this dataset:

COLUMN	HEADER			EXPLANATION

1.		GROUP/SHIP:			AOML_Brown for all underway data from the Ron Brown.

2.		CRUISE_DESIGNATION:	Cruise ID (e.g., RBYYYYnn where RB = Ron Brown, YYYY

= the four digit year, and nn = the cruise number for

that year).

3.		JD_GMT:			Decimal year day.

4.		DATE_DDMMYYYY:		GMT date.  The date format has been changed to comply

with the IOCCP recommendations.

5.		TIME_HH:MM:SS:		GMT time.

6.		LAT_DEC_DEGREE:		Latitude in decimal degrees (negative values are in 							the southern hemisphere).

7.		LONG_DEC_DEGREE:		Longitude in decimal degrees (negative values are in 							the western hemisphere).

8.		xCO2W_PPM:			Mole fraction of CO2 (dry) in the equilibrator

headspace at equilibrator temperature (Teq) in parts

per million.

9.		xCO2A_PPM:			Mole fraction of CO2 in air in parts per million.

10.		EqTEMP_C:			Temperature in equilibrator water in degrees

centigade.  Temperature in equilibrator measured with

a calibrated thermistor.

11.		PRES_EQUIL_hPa:		Barometric pressure in the lab in hectopascals (1

hectopascal = 1 millibar).

12.		SST(TSG)_C:			Temperature from the ship's thermosalinograph in

degrees centigrade.

13.		SAL(TSG)_PERMIL:		Salinity from the ship's thermosalinograph on the

Practical Salinity Scale.

14.		fCO2w,eq:			Fugacity of CO2 in the equilibrator in

microatmospheres calculated as outlined below.

15.		fCO2W@SST_uatm:		Fugacity of CO2 in sea water in microatmospheres

calculated as outlined below.

16.		fCO2A_uATM:			Fugacity of CO2 in air in microatmospheres

calculated as outlined below.

17.		dfCO2_uatm:			Sea water fCO2 - air fCO2 in microatmospheres.  This

uses the average air value for the current hour.

The following fields have been QC'ed by the CO2 group:

GROUP/SHIP

CRUISE_DESIGNATION

JD_GMT

DATE_DDMMYYYY

TIME_HH:MM:SS

LAT_DEC_DEGREE

LONG_DEC_DEGREE

xCO2W_PPM

xCO2A_PPM

EqTEMP_C

PRES_EQUIL_hPa

fCO2w,eq

fCO2W@SST_uatm

fCO2A_uATM

dfCO2_uatm

The following fields are from the ship's onboard systems and the quality of this

data cannot be verified:

SST(TSG)_C

Sal(TSG)_Permil

Narrative description of system design:

CO2 ANALYTICAL SYSTEM:

The concentration of carbon dioxide (CO2) in surface ocean water is determined

by measuring the concentration of CO2 in gas that is in contact with the water.

Surface water is pumped ~ 100 m through 7/8" Teflon tubing from an inlet

in the ship's bow to the equilibration chamber.  Water comes from the bow

intake ~4.2 m below the water line and the TSG is located close to the inlet.

When the SST is below about 20 oC, friction in the pipes and from the pump cause

heating and the Teq is higher than SST.  When the SST is higher than about 25 oC,

the ship’s air conditioning cools the water and the Teq is lower than SST.

The equilibration chamber has an enclosed volume of gas, or headspace, and a pool

of seawater that continuously overflows to a drain.  As the water flows through the

chamber, the dissolved gases (like CO2) partition between the water and the

headspace.  At equilibrium, the ratio of CO2 in the water and in the headspace is

influenced most by temperature, and that relationship is known.  By measuring

the concentration of CO2 in the headspace and the temperature in the chamber,

the partial pressure (or fugacity) of CO2 in the surface water can be calculated.

INSTRUMENT DESCRIPTION

The general principle of instrumental design can be found in Wanninkhof and Thoning

(1993), Ho et al. (1995), and Feely et al. (1999).  The concentration of CO2 in the

headspace gas is measured using the adsorption of infrared (IR) radiation, which

results from changes in the rotational and vibrational energy state of the CO2

molecule.  The LI-COR detector passes IR radiation through two 6" cells.  The

reference cell is flushed with a gas of known CO2 concentration.  The sample cell

is flushed with the headspace gas.  A vacuum-sealed, heated filament is the

broadband IR source.  The IR radiation alternates between the two cells via a

chopping shutter disc. An optical filter selects an adsorption band specific

for CO2 (4.26 micron) to reach the detector.  The solid state (lead selenide)

detector is kept at -12 degrees °C for excellent stability and low signal

noise (less than 0.2 ppm).

Several steps are taken to reduce interferences and to increase the accuracy

of the measurements.  After the equilibration chamber, the headspace travels

through a drying trap to remove water vapor.  During each analysis, the

headspace gas is compared to a reference gas of known concentration.  To

improve the accuracy of the measurements, three different gaseous standards

for CO2 are analyzed once an hour instead of the headspace gas.

Analyzer:  LI-COR 6251 (analog output) infrared (IR) analyzer.

Method of Analysis:  Differential analyses relative to the low standard.  Measures

dried equilibrator headspace gas.  Gas flow is stopped prior to IR readings.

Drying Method:  The equilibrator headspace sample gas first goes through a glass

condenser cooled to ~ 5 oC.  The sample and standard gases pass through a short

column of magnesium perchlorate before reaching the analyzer.

Equilibrator (setup, size, flows):  The equilibrator is based on a design by R.

Weiss and was fabricated from a plexiglass housing with ~8 L water reservoir and

~16 L gaseous headspace.  Water flow rate is ~11 L/min.  Headspace recirculation

rate is ~200 ml/min.

Additional sensors:

Thermistor mounted in the bottom of the equilibrator.

Setra Barometer Model 370

YSI Model 600R thermosalinograph with temperature, salinity, and dissolved

oxygen sensors.  This TSG is mounted in the Hydro lab sink near the

equilibrator and the two are teed off the uncontaminated seawater feed.

The dissolved oxygen measurements are not reported in the final data file.

Narrative statement identifying measurement method for each required parameter:

CALCULATIONS:

The mixing ratios of ambient air and equilibrated headspace air are calculated

by fitting a second-order polynomial through the hourly averaged millivolt

response of the detector versus mixing ratios of the standards.  Mixing ratios

of dried equilibrated headspace and air are converted to fugacity of CO2 in

surface seawater and water saturated air in order to determine the fCO2.

For ambient air and equilibrator headspace, the fCO2a (or fCO2eq) is calculated

assuming 100% water vapor content:

fCO2eq = xCO2eq(P-pH2O)exp(B11+2*d12)P/RT

where fCO2eq is the fugacity in the equilibrator, pH2O is the water vapor

pressure at the sea surface temperature, P is the atmospheric pressure (in atm),

T is the SST or equilibrator temperature (in K) and R is the ideal gas constant

(82.057 cm^3·atm·deg^-1·mol^-1).  The exponential term is the fugacity correction

where B11 is the second virial coefficient of pure CO2

B11 = -1636.75 + 12.0408T - 0.032795T^2 + 3.16528E-5 T^3

and d12 = 57.7 - 0.118 T is the correction for an air-CO2 mixture in units of

cm^3·mol^-1 (Weiss, 1974).

The calculation for the fugacity at SST involves a temperature correction term

for the increase of fCO2 due to heating of the water from passing through the

pump and through 5 cm ID PVC tubing within the ship.  The empirical temperature

correction from equilibrator temperature to SST is:

fCO2(SST) = fCO2(eq) /

Exp ((Teq-SST) * [0.03107 – 2.7851E-4 * Teq – 1.8391E-3 * ln(fco2eq * 1.0E-6)])

where SST is sea surface temperature and Teq is the equilibrator temperature in

degrees °C.

Sampling Cycle:

The system runs on an hourly cycle during which 3 standard gases, 3 air

samples from the bow tower and 8 surface water samples (from the

equilibrator head space) are analyzed on the following schedule:

Mins. after hour		Sample

4 				Low Standard

8 				Mid Standard

12 				High Standard

16.5				Water

21 				Water

25.5				Water

30				Water

34				Air

38				Air

42				Air

46.5				Water

51				Water

55.5				Water

60				Water

NOTES ON DATA:

Columns have a default value of –999.99 in case of instrument malfunction,

erroneous readings or missing data.  Furthermore, if a suspicious xCO2 value,

pressure or temperature value is encountered, the fCO2 is not calculated.

Analytical Instrument Manufacturer/Model:

The Ron Brown system (version 2.6) was built by Craig Neill in 1999.  The analyzer 	is a LI-COR 6251 (analog output) infrared analyzer.

Standard Gases and Reference Gas:  The three standard gases came from CMDL

in Boulder and are directly traceable to the WMO scale.  While individual data

points above the high standard gas concentration or below the low standard gas

concentration may not be accurate, the general trends should be indicative of the

seawater chemistry.

Description of any additional environmental control:

The system is located in the Hydro Lab of the Ron Brown.  The room is

air-conditioned with little temperature fluctuation.

Resolution of measurement:

The resolution of the instrument is better than 0.1 ppm.

Estimated overall uncertainty of measurement:

The xCO2eq measurements are believed accurate to 0.1 ppm.  The

fCO2@SST measurements are believed to be precise to 0.2 ppm.

List of calibration gases used:

The standards used during the 2002 field season were:

STANDARD		TANK #	CONCENTRATION	VENDOR

Low			CA04403	291.58		ESRL

Low			CC114999	275.63		ESRL

Low			CA01433	293.73		ESRL

Mid			CA02901	339.57		ESRL

Mid			CA05098	358.87		ESRL

High/Mid		CC71655	423.28		ESRL

High			CA03079	431.30		ESRL

High			CA03888	524.99		ESRL

High			CC71588	531.98		ESRL

Traceability to an internationally recognized scale (including date/place of last calibration made):

All standards are obtained from NOAA/CMDL, now called the Global

Monitoring Division of the Earth System Research Laboratory and are directly

traceable to WMO scale.

Uncertainty of assigned value of each calibration gas:

The uncertainty based on pre and post cruise calibrations is less

than 0.05 ppm.

Pressure/Temperature/Salinity:

For information about the ship’s thermosalinograph, contact Chief Survey Tech

Jonathan Shannahoff at jonathan.shannahoff@noaa.gov.

Units:

All xCO2 values are reported in parts per million (ppm) and fCO2 values

are reported in microatmospheres (uatm) assuming 100% humidity at the

equilibrator temperature.

Bibliography:

DOE (1994). Handbook of methods for the analysis of the various parameters of the

carbon dioxide system in sea water; version 2. DOE.

Feely, R. A., R. Wanninkhof, H. B. Milburn, C. E. Cosca, M. Stapp and P. P. Murphy

(1998). A new automated underway system for making high precision pCO2

measurements onboard research ships. Analytica Chim. Acta 377: 185-191.

Ho, D. T., R. Wanninkhof, J. Masters, R. A. Feely and C. E. Cosca (1997).

Measurement of underway fCO2 in the Eastern Equatorial Pacific on NOAA

ships BALDRIGE and DISCOVERER, NOAA data report ERL AOML-30, 52 pp.,

NTIS Springfield.

Wanninkhof, R. and K. Thoning (1993). Measurement of fugacity of CO2 in surface

water using continuous and discrete sampling methods. Mar. Chem. 44(2-4):

189-205.

Weiss, R. F. (1970). The solubility of nitrogen, oxygen and argon in water and

seawater. Deep-Sea Research 17: 721-735.

Weiss, R. F. (1974). Carbon dioxide in water and seawater: the solubility of

a non-ideal gas. Mar. Chem. 2: 203-215.

Weiss, R. F., R. A. Jahnke and C. D. Keeling (1982). Seasonal effects of

temperature and salinity on the partial pressure of CO2 in seawater.

Nature 300: 511-513.

Comments related to all 2002 data:

1.	xCO2 values outside the range of the standard gases (i.e. below the low standard or	above the high standard) are not as accurate as values within the range.  However,	the general trends should be indicative of the seawater chemistry.

2.	The standard gases for the first 2 cruises (RB200201T & RB200201) were 291.58 ppm, 	339.57 ppm, and 431.30 	ppm.  For the next 2 cruises (RB200202 & RB200203) the 	standards were 275.63 ppm, 339.57 ppm, and 431.30 ppm.  For cruise RB200204T the 	standards were 275.63 ppm, 339.57 ppm, and 524.99 ppm.  For cruises RB200204, 	RB200204, RB200204, RB200204, RB200204, RB200204, and RB200204 the standards were 	293.73 ppm, 358.87 ppm, and 423.28 ppm.  For cruise RB200209 the standards were 	293.73 ppm, 358.87 ppm, and 531.98 ppm.

Comments related to the individual legs:

RB200201T:	1. For the first 15 hours of the leg, there was no flow in either the 				mid standard or the high standard.  For this time period I derived 				voltage values for mid and high standards by using a 2nd order curve 				fit between the low standard voltage and the other two based on the 				next 42 hours of values.  For the mid standard (Std 2) I used the 				equation S2V = -19.141 * S1V^2 + 2.8042 * S1V + 0.2021 (R^2 = 0.9974) 				for the mid standard and S3V = -57.415 * S1V^2 + 6.2084 * S1V + 0.5545 				(R^2 = 0.9819) for the high standard.  Water values for this period 				were in the range of 335 - 365 ppm and air values were in the range of 				377 - 381 ppm, both of which are close to the mid standard value of 340 			ppm.  For this reason I believe that the values in the file are within 				+/- 3 ppm of the actual value.

2. The feed from the ship's computer system was down for about 12 hours 			on Jan. 21 - Jan. 22 and for that period the system recorded no SST or 				salinity from the ship's TSG.  I derived values for SST from the 					equilibrator thermistor and for salinity from the YSI probe for this 				time period.  For SST, I used the equation SST = EqT - 0.11 based on 				approximately 42 hours worth of values where SST was above 27 degrees 				C.  For salinity, I used the equation S = YSI salinity - 0.44 based on 				all other data from this leg.

3. There were 3 data dropouts of approximately 30, 36, and 42 hours 				where the ship was not allowed to sample in certain territorial waters.

RB200201:	No problems of note.

RB200202:	1. The low standard cylinder was changed on March 4 to 275.63.  Some 				values were removed at this time because of the change.  After the 				change, the air values increased by ~2 ppm.  We have been unable to 				determine the cause of this and have left the values in the data file.

2. The equilibrator thermistor was reading incorrectly up until the 				time of the cylinder change.  I have substituted a derived value for 				equilibrator temperature for the first part of the cruise using the YSI 			TSG temperature as follows:  EqT = 0.9401 * YSI T + 1.334 (r^2 = 					0.8904).  This equation was derived from values after the cylinder 				change where comparisons with SST and the YSI indicate the thermistor 				was reading correctly.

RB200203:	No problems of note.

RB200204T:	1. Air values on this leg were high - in the range 382 - 386 ppm.  No 				reason for the high readings could be found but they should be 					considered suspect.  Normal air values for this region and time of year 			should be a little over 370 ppm.

RB200204:	No problems of note.

RB200205A:	1. Seawater was turned off from July 15 at 0915 to July 16 at 0015 				(GMT) due to dirty water in New York harbor.  All surface water samples 			during that period have been removed.

2. Coastal air transects resulted in air concentration values varying 				from 350-405 ppm because of air masses originating from land.

3. The delta fCO2 values were determined using the highly variable air 				values.

RB200205B:	1. Seawater was turned off on August 9 from 0150 to 1830 (GMT) due to 				dirty water.  All surface water samples during that period have been 				removed.

2. Coastal air transects resulted in air concentration values varying 				from 355-440 ppm because of air masses originating from land.

3. The delta fCO2 values were determined using the highly variable air 				values.

RB200206:	1. Seawater was turned off on August 19 from 1355 to 2046 (GMT) when 				the ship stopped in Miami.  All surface water and air samples during 				that period have been removed.

RB200207:	1. Seawater was turned off from September 1 at 1120 to September 3 at 				2046 (GMT) when the ship entered New York harbor.  All surface water 				and air samples during that period have been removed.

2. There were peaks in the air values on September 5 and September 11 				which have been removed from the data file.  We cannot attribute these 				to contamination by stack gases and assume they were caused by 					pollution from sources on land.  Anyone interested in this data should 				contact Bob Castle at Robert.castle@noaa.gov.

RB200208T:	1. On September 23, gas flow in the air phase dropped below 30 ml/min 				due to water in the line.  Many air values were removed for that 					reason.

RB200208:	1. The system was down from 9/25 at 2108 to 9/26 at 1145 (GMT) due to a 			suspected software glitch.  No data was recorded during this time.

2. Gas flow in the air phase dropped to 0 on 9/25 at 1630 due to water 				clogging the line.  From that time to the end of the file I 					extrapolated the average air value of 369.5 ppm.

RB200209:	1. The TSG malfunctioned on this cruise, resulting in high variance in 				SST and salinity.  The salinity data was particularly noisy.  I believe 			this was at least in part due to the noise in the temperature signal.  				To check the values, I compared them to the data from 45 CTD casts, 				using the upcast values at 2 meters depth.  The temperature values were 			close, but the CTD salinities were higher than the TSG values.  For the 			values in this data file, I did a 15 minute running average of the TSG 				SST and salinity recorded by our system.  I then matched these values 				up with the CTD temperature and salinity.  For SST, I used the 15 				minute average with no adjustments since the average difference of the 				values was -0.01416 degrees with a standard deviation of 0.1237 for n = 			44 CTDs.  For salinity, I corrected the 15 minute averaged TSG values 				by adding 0.194 to them.  This is the average difference (CTD sal - TSG 			sal) for n = 45 CTDs, with a standard deviation of 0.034.  All fCO2 				values are computed using the averaged and adjusted TSG data.