R/V Ronald H. Brown METADATA - 2004

Class of Data:  Surface ocean and atmospheric carbon dioxide concentrations

Dataset Identifier:  R/V Ronald H. Brown

One File:  RHB2004

Statement of how to cite dataset:

Ron Brown website: http://www.aoml.noaa.gov/ocd/gcc/rvbrown_data2004.php

These data are made freely available to the public and the scientific community

in the belief that their wide dissemination will lead to greater understanding

and new scientific insights.  The availability of these data does not

constitute publication of the data.  We rely on the ethics and integrity of

the user to assure that AOML receives fair credit for our work.  Please send

manuscripts using this data to AOML for review before they are submitted

for publication so we can insure that the quality and limitations of the data

are accurately represented.

Measurement platform identifier:  NOAA research vessel Ronald H. Brown (R104)

Cruise Information:

The Ron Brown conducted 11 major cruises in the Atlantic and eastern Pacific

Oceans for a total of 14 legs.

Project Information:

The system was operated by personnel from AOML or PMEL (Pacific Marine 	Environmental Laboratory) or by the Ron Brown’s Chief Survey Tech, Jonathan 	Shannahoff.  The work was sponsored by the Underway pCO2 on Ships project of the 	NOAA climate program.

Scientist responsible for technical quality of dataset:

Rik Wanninkhof

NOAA/AOML/Ocean Chemistry Division

4301 Rickenbacker Causeway

Miami, Florida 33149

Rik.Wanninkhof@noaa.gov

Contact person for this dataset:

Bob Castle

NOAA/AOML/Ocean Chemistry Division

4301 Rickenbacker Causeway

Miami, Florida 33149

Robert.Castle@noaa.gov

Timestamp for initial submission of dataset:  11/18/09

Timestamp for the most recent update of dataset:  11/18/09

Timestamp period the dataset refers to: 2/12/2004 – 12/22/2004

Geographic area the dataset refers to:

35 S to 50 N

115 W to 15 W

2004 Cruises:

RB200401 –	NTAS Mooring

Charleston, SC to Bridgetown, Barbados

February 12, 2004 to February 25, 2004

Chief Scientist – Al Plueddemann

Operator – Jonathan Shannahoff

RB200402A -	Saharan Dust Leg A

Bridgetown, Barbados to Las Palmas, Canary Islands

March 1, 2004 to March 15, 2004

Chief Scientist – Pablo Clemente-Colon

Operator – Jonathan Shannahoff

RB200402B -	Saharan Dust Leg B

Las Palmas, Canary Islands to San Juan, Puerto Rico

March 17, 2004 to March 26, 2004

Chief Scientist – Pablo Clemente-Colon

Operator – Jonathan Shannahoff

RB200403 -	Windward Passage

San Juan, Puerto Rico to Charleston, SC

March 29, 2004 to April 13, 2004

Chief Scientist – Ryan Smith

Operator – Jonathan Shannahoff

RB200404T -	Ocean Explorations Transit

Charleston, SC to Woods Hole, MA

April 30, 2004 to May 3, 2004

Chief Scientist – Jeff Hare

Operator – Jonathan Shannahoff

RB200404A -	Ocean Explorations Mountains o’ the Sea II

Woods Hole, MA, to Boston, MA

May 9, 2004 to May 24, 2004

Chief Scientist – Les Watling

Operator – Jonathan Shannahoff

RB200405 -	Ocean Explorations Titanic

Boston, MA to Boston, MA

May 27, 2004 to June 10, 2004

Chief Scientist – Robert Ballard

Operator – Jonathan Shannahoff

RB200406 -	NURP Deep Sea Corals

Boston, MA to Portsmouth, NH

June 17, 2004 to June 28, 2004

Chief Scientist – Les Watling

Operator – Jonathan Shannahoff

RB200407A -	New England Air Quality Study Leg A

Portsmouth, NH to Portsmouth, NH

July 5, 2004 to July 23, 2004

Chief Scientist – Tim Bates

Operator – Jonathan Shannahoff

RB200407B -	New England Air Quality Study Leg B

Portsmouth, NH to Portsmouth, NH

July 26, 2004 to August 12, 2004

Chief Scientist – Tim Bates

Operator – Jonathan Shannahoff

RB200408 -	Windward Passage

Charleston, SC to Miami, FL

September 22, 2004 to October 3, 2004

Chief Scientist – Chris Meinen

Operator – Jonathan Shannahoff

RB200409 -	Windward Passage

Miami, FL to San Juan, Puerto Rico

October 10, 2004 to October 18, 2004

Chief Scientist – Bill Johns

Operator – Jonathan Shannahoff

RB200410 -	TAO

Rodman, Panama to Arica, Chile

October 29, 2004 to November 28, 2004

Chief Scientist – David Zimmerman

Operator – Jonathan Shannahoff

RB200411 -	STRATUS Mooring

Arica, Chile to Valparaiso, Chile

December 5, 2004 to December 22, 2004

Chief Scientist – Robert Weller

Operator – Jonathan Shannahoff

List of variables included in this dataset:

COLUMN	HEADER			EXPLANATION

1.		GROUP/SHIP:			AOML_Brown for all underway data from the Ron Brown.

2.		CRUISE_DESIGNATION:	Cruise ID (e.g., RBYYYYnn where RB = Ron Brown, YYYY

= the four digit year, and nn = the cruise number for

that year).

3.		JD_GMT:			Decimal year day.

4.		DATE_DDMMYYYY:		GMT date.  The date format has been changed to comply

with the IOCCP recommendations.

5.		TIME_HH:MM:SS:		GMT time.

6.		LAT_DEC_DEGREE:		Latitude in decimal degrees (negative values are in 							the southern hemisphere).

7.		LONG_DEC_DEGREE:		Longitude in decimal degrees (negative values are in 							the western hemisphere).

8.		xCO2W_PPM:			Mole fraction of CO2 (dry) in the equilibrator

headspace at equilibrator temperature (Teq) in parts

per million.

9.		xCO2A_PPM:			Mole fraction of CO2 in air in parts per million.

10.		PRES_EQUIL_hPa:		Barometric pressure in the lab in hectopascals (1

hectopascal = 1 millibar).

11.		PRES_SEALEVEL_hPa:	Barometric pressure corrected to sea level from the 							ship’s barometer in hectopascals (1 hectopascal = 1 							millibar).

12.		EqTEMP_C:			Temperature in equilibrator water in degrees

centigade.  Temperature in equilibrator measured with

a calibrated thermistor.

13.		SST(TSG)_C:			Temperature from the ship's thermosalinograph in

degrees centigrade.

14.		SAL(TSG)_PERMIL:		Salinity from the ship's thermosalinograph on the

Practical Salinity Scale.

15.		WATER_FLOW_L/MIN:		Water flow rate through the equilibrator in liters 							per minute.

16.		GAS_FLOW_IR_ML/MIN:	Gas flow through the sample cell of the Licor IR 							analyzer in milliliters per minute.

17.		TEMP_IR_C:			Temperature in the Licor sample cell in degrees 							centigrade.

18.		PRES_IR_hPa:		Barometric pressure in the lab in hectopascals (1

hectopascal = 1 millibar).  The Licor in this system 							does not include a pressure sensor so this field is 							the same as # 10 above.

19.		SHIP_HEADING_TRUE_DEGREE:	Ship’s heading in true degrees from the ship’s 								scientific computing system.

20.		SHIP_SPEED_KNOT:		Ship’s speed in knots from the ship’s scientific 							computing system.

21.		WIND_DIR_REL_DEGREE:	Relative wind direction in degrees from the ship’s 							scientific computing system.

22.		WIND_SPEED_REL_M/S:	Relative wind speed in meters per second from the 							ship’s scientific computing system.

23.		fCO2W@SST_uatm:		Fugacity of CO2 in sea water in microatmospheres

calculated as outlined below.

24.		QC_FLAG_WATER:		Quality control flag for fCO2W@SST measurement.  2 = 							good, 3 = questionable, 4 = bad.

25.		fCO2A_uATM:			Fugacity of CO2 in air in microatmospheres

calculated as outlined below.

24.		QC_FLAG_AIR:		Quality control flag for fCO2A measurement.  2 = 							good, 3 = questionable, 4 = bad.

27.		dfCO2_uATM:			Sea water fCO2 - air fCO2 in microatmospheres.  This

uses the average air value for the current hour.

28.		FLUORO_uG/l:		Measurement from the ship’s Turner 10AU fluorometer 							in micrograms per liter.

29.		WIND_SPEED_TRUE_M/S:	True wind speed in meters per second from the 								ship’s scientific computing system.

30.		WIND_DIR_TRUE_DEGREE:	True wind direction in degrees from the ship’s 								scientific computing system.

31.		AIR_TEMP_C:			Outside air temperature from the ship’s scientific 							computing system.

The following fields have been QC'ed by the CO2 group:

GROUP/SHIP

CRUISE_DESIGNATION

JD_GMT

DATE_DDMMYYYY

TIME_HH:MM:SS

LAT_DEC_DEGREE

LONG_DEC_DEGREE

xCO2W_PPM

xCO2A_PPM

EqTEMP_C

PRES_EQUIL_hPa

WATER_FLOW_L/MIN

GAS_FLOW_L/MIN

TEMP_IR_C

PRES_IR_hPa

fCO2W@SST_uatm

fCO2A_uATM

dfCO2_uatm

The following fields are from the ship's onboard systems and the quality of this

data cannot be verified:

SST(TSG)_C

Sal(TSG)_Permil

PRES_SEALEVEL_hPa

SHIP_HEADING_TRUE_DEGREE

SHIP_SPEED_KNOT

WIND_DIR_REL_DEGREE

WIND_SPEED_REL_M/S

FLUORO_uG/l

WIND_SPEED_TRUE_M/S

WIND_DIR_TRUE_DEGREE

AIR_TEMP_C

Narrative description of system design:

CO2 ANALYTICAL SYSTEM:

The concentration of carbon dioxide (CO2) in surface ocean water is determined

by measuring the concentration of CO2 in gas that is in contact with the water.

Surface water is pumped ~ 100 m through 7/8" Teflon tubing from an inlet

in the ship's bow to the equilibration chamber.  Water comes from the bow

intake ~4.2 m below the water line and the TSG is located close to the inlet.

When the SST is below about 20 oC, friction in the pipes and from the pump cause

heating and the Teq is higher than SST.  When the SST is higher than about 25 oC,

the ship’s air conditioning cools the water and the Teq is lower than SST.

The equilibration chamber has an enclosed volume of gas, or headspace, and a pool

of seawater that continuously overflows to a drain.  As the water flows through the

chamber, the dissolved gases (like CO2) partition between the water and the

headspace.  At equilibrium, the ratio of CO2 in the water and in the headspace is

influenced most by temperature, and that relationship is known.  By measuring

the concentration of CO2 in the headspace and the temperature in the chamber,

the partial pressure (or fugacity) of CO2 in the surface water can be calculated.

INSTRUMENT DESCRIPTION

The general principle of instrumental design can be found in Wanninkhof and Thoning

(1993), Ho et al. (1995), and Feely et al. (1999).  The concentration of CO2 in the

headspace gas is measured using the adsorption of infrared (IR) radiation, which

results from changes in the rotational and vibrational energy state of the CO2

molecule.  The LI-COR detector passes IR radiation through two 6" cells.  The

reference cell is flushed with a gas of known CO2 concentration.  The sample cell

is flushed with the headspace gas.  A vacuum-sealed, heated filament is the

broadband IR source.  The IR radiation alternates between the two cells via a

chopping shutter disc. An optical filter selects an adsorption band specific

for CO2 (4.26 micron) to reach the detector.  The solid state (lead selenide)

detector is kept at -12 degrees °C for excellent stability and low signal

noise (less than 0.2 ppm).

Several steps are taken to reduce interferences and to increase the accuracy

of the measurements.  After the equilibration chamber, the headspace travels

through a drying trap to remove water vapor.  During each analysis, the

headspace gas is compared to a reference gas of known concentration.  To

improve the accuracy of the measurements, three different gaseous standards

for CO2 are analyzed once an hour instead of the headspace gas.

Analyzer:  LI-COR 6251 (analog output) infrared (IR) analyzer.

Method of Analysis:  Differential analyses relative to the low standard.  Measures

dried equilibrator headspace gas.  Gas flow is stopped prior to IR readings.

Drying Method:  The equilibrator headspace sample gas first goes through a glass

condenser cooled to ~ 5 oC.  The sample and standard gases pass through a short

column of magnesium perchlorate before reaching the analyzer.

Equilibrator (setup, size, flows):  The equilibrator is based on a design by R.

Weiss and was fabricated from a plexiglass housing with ~8 L water reservoir and

~16 L gaseous headspace.  Water flow rate is ~11 L/min.  Headspace recirculation

rate is ~200 ml/min.

Additional sensors:

Thermistor mounted in the bottom of the equilibrator.

Setra Barometer Model 370

Seabird Micro TSG Model SBE-45 with temperature and salinity sensors.  This TSG is 			mounted in the Hydro lab sink near the equilibrator and the two are teed off 			the uncontaminated seawater feed.

Narrative statement identifying measurement method for each required parameter:

CALCULATIONS:

The mixing ratios of ambient air and equilibrated headspace air are calculated

by fitting a second-order polynomial through the hourly averaged millivolt

response of the detector versus mixing ratios of the standards.  Mixing ratios

of dried equilibrated headspace and air are converted to fugacity of CO2 in

surface seawater and water saturated air in order to determine the fCO2.

For ambient air and equilibrator headspace, the fCO2a (or fCO2eq) is calculated

assuming 100% water vapor content:

fCO2eq = xCO2eq(P-pH2O)exp(B11+2*d12)P/RT

where fCO2eq is the fugacity in the equilibrator, pH2O is the water vapor

pressure at the sea surface temperature, P is the atmospheric pressure (in atm),

T is the SST or equilibrator temperature (in K) and R is the ideal gas constant

(82.057 cm^3·atm·deg^-1·mol^-1).  The exponential term is the fugacity correction

where B11 is the second virial coefficient of pure CO2

B11 = -1636.75 + 12.0408T - 0.032795T^2 + 3.16528E-5 T^3

and d12 = 57.7 - 0.118 T is the correction for an air-CO2 mixture in units of

cm^3·mol^-1 (Weiss, 1974).

The calculation for the fugacity at SST involves a temperature correction term

for the increase of fCO2 due to heating of the water from passing through the

pump and through 5 cm ID PVC tubing within the ship.  The empirical temperature

correction from equilibrator temperature to SST is:

fCO2(SST) = fCO2(eq) /

Exp ((Teq-SST) * [0.03107 – 2.7851E-4 * Teq – 1.8391E-3 * ln(fco2eq * 1.0E-6)])

where SST is sea surface temperature and Teq is the equilibrator temperature in

degrees °C.

Sampling Cycle:

The system runs on an hourly cycle during which 3 standard gases, 3 air

samples from the bow tower and 8 surface water samples (from the

equilibrator head space) are analyzed on the following schedule:

Mins. after hour		Sample

4 				Low Standard

8 				Mid Standard

12 				High Standard

16.5				Water

21 				Water

25.5				Water

30				Water

34				Air

38				Air

42				Air

46.5				Water

51				Water

55.5				Water

60				Water

NOTES ON DATA:

Columns have a default value of –999.99 in case of instrument malfunction,

erroneous readings or missing data.  Furthermore, if a suspicious xCO2 value,

pressure or temperature value is encountered, the fCO2 is not calculated.

Analytical Instrument Manufacturer/Model:

The Ron Brown system (version 2.6) was built by Craig Neill in 1999.  The analyzer 	is a LI-COR 6251 (analog output) infrared analyzer.

Standard Gases and Reference Gas:  The three standard gases came from CMDL

in Boulder and are directly traceable to the WMO scale.  While individual data

points above the high standard gas concentration or below the low standard gas

concentration may not be accurate, the general trends should be indicative of the

seawater chemistry.

Description of any additional environmental control:

The system is located in the Hydro Lab of the Ron Brown.  The room is

air-conditioned with little temperature fluctuation.

Resolution of measurement:

The resolution of the instrument is better than 0.1 ppm.

Estimated overall uncertainty of measurement:

The xCO2eq measurements are believed accurate to 0.1 ppm.  The

fCO2@SST measurements are believed to be precise to 0.2 ppm.

List of calibration gases used:

The standards used during the 2004 field season were:

STANDARD		TANK #	CONCENTRATION	VENDOR

Low			CA01979	296.73		ESRL

Low			CA05337	294.06		ESRL

Low			CA05395	315.25		ESRL

Mid			CC91287	357.18		ESRL

Mid,High		CA05344	411.42		ESRL

High			CC71655	423.28		ESRL

High			CC71588	531.98		ESRL

Traceability to an internationally recognized scale (including date/place of last calibration made):

All standards are obtained from NOAA/CMDL, now called the Global

Monitoring Division of the Earth System Research Laboratory and are directly

traceable to WMO scale.

Uncertainty of assigned value of each calibration gas:

The uncertainty based on pre and post cruise calibrations is less

than 0.05 ppm.

Pressure/Temperature/Salinity:

For information about the ship’s thermosalinograph or other sensors, contact Chief 	Survey Tech Jonathan Shannahoff at jonathan.shannahoff@noaa.gov.

Units:

All xCO2 values are reported in parts per million (ppm) and fCO2 values

are reported in microatmospheres (uatm) assuming 100% humidity at the

equilibrator temperature.

Bibliography:

DOE (1994). Handbook of methods for the analysis of the various parameters of the

carbon dioxide system in sea water; version 2. DOE.

Feely, R. A., R. Wanninkhof, H. B. Milburn, C. E. Cosca, M. Stapp and P. P. Murphy

(1998). A new automated underway system for making high precision pCO2

measurements onboard research ships. Analytica Chim. Acta 377: 185-191.

Ho, D. T., R. Wanninkhof, J. Masters, R. A. Feely and C. E. Cosca (1997).

Measurement of underway fCO2 in the Eastern Equatorial Pacific on NOAA

ships BALDRIGE and DISCOVERER, NOAA data report ERL AOML-30, 52 pp.,

NTIS Springfield.

Wanninkhof, R. and K. Thoning (1993). Measurement of fugacity of CO2 in surface

water using continuous and discrete sampling methods. Mar. Chem. 44(2-4):

189-205.

Weiss, R. F. (1970). The solubility of nitrogen, oxygen and argon in water and

seawater. Deep-Sea Research 17: 721-735.

Weiss, R. F. (1974). Carbon dioxide in water and seawater: the solubility of

a non-ideal gas. Mar. Chem. 2: 203-215.

Weiss, R. F., R. A. Jahnke and C. D. Keeling (1982). Seasonal effects of

temperature and salinity on the partial pressure of CO2 in seawater.

Nature 300: 511-513.

Comments related to all 2004 data:

1.	xCO2 values outside the range of the standard gases (i.e. below the low standard or	above the high standard) are not as accurate as values within the range.  However,	the general trends should be indicative of the seawater chemistry.

2.	The standard gases for the first 4 cruises (RB200401-RB200403) were 296.37 ppm, 	357.18 ppm, and 423.28 ppm.  The standards for the next 5 cruises (RB200404T-	RB200407B) were 294.06 ppm, 357.18 ppm, and 423.28 ppm.  The standards for cruise 	RB200408 were 315.25 ppm, 357.18 ppm, and 411.42 ppm.  The standards for the last 3 	cruises (RB200409-RB200411) were 315.25 ppm, 411.42 ppm, and 531.98 ppm.

3.	Pressure from the ship’s barometer corrected to sea level, ship speed over ground 	(SOG), and outside air temperature were not recorded for the first 3 cruises 	(RB200401-RB200402B) or for cruise RB200404A.  Ship heading (COG) was not recorded 	for the first 6 cruises (RB200401-RB200404A).

4.	At the beginning of the 2004 field season, two mistakes were made in setting up the 	system.  The first error was in the plumbing of the equilibrator and may result in 		slight inaccuracies in the water xCO2 values and some additional variance in the 		values.  The increased variance should be less than 2 ppm and probably less than 1 		ppm.  The second error was in plumbing the water flow meter backwards and resulted 		in flow values of approximately 3.5 liters per minute in the data file.  I believe 		that the true flow was in the normal range of approximately 11 liters per minute.  	This problem should have no effect on xCO2 values.  Both of these problems were 	corrected after the fourth cruise (RB200403).

5.	The YSI thermosalinograph was replaced with an SBE micro TSG (model SBE-45) from 	Seabird that includes temperature and conductivity sensors.  The data from this 	unit is only used for quality control checks and is not included in the final data 	file.  It can be obtained from Bob Castle (robert.castle@noaa.gov) if necessary.

Comments related to the individual legs:

RB200401:	1. The first 15 hours of data was removed (given a QC flag of 4) due to 			instability of the voltage response of the standard gases.

RB200402A:	No problems of note (but see #4 above).

RB200402B:	No problems of note (but see #4 above).

RB200403:	1. The last 19 hours of data (from April 12 @ 1845 to April 13 @ 1325) 				was removed due to insufficient flow of gases in all phases.  Licor 				voltages were also bad.

2. A group from the University of South Florida led by Zhaohui Aleck 				Wang teed off of our standard gases and air line feed during this 				cruise to test a new pCO2 instrument they are developing.  We believe 				this had no deleterious effect on the data collected.

RB200404T:	1. Beginning on May 1 at 12:16 to May 2 at 10:00 there is unusual high 				variability in the water xCO2 values.  I believe these values may be 				real but have given them a QC flag of 3 because there is also high 				variability in salinity and SST during this period.

RB200404A:	No problems of note.

RB200405:	1. The gas flow in both the air and water phases dropped to zero during 			the time periods May 28 from 1215 to 1300 and from 1534 to 1804, June 6 			from 0617 to 0705 and from 0817 to 1205 and from 1526 to 1705, and on 				June 7 from 0817 to 1805.  All air and water values during these time 				periods were removed.

2. Sea water flow dropped to zero during the periods June 9 from 0522 				to 0830 and on June 10 from 0530 to 1305 and from 1722 to 1836.  All 				water values during these time periods were removed.

3. The ship's salinity sensor readings drifted down relative to our 				Seabird sensor near the equilibrator on four occasions - three at the 				beginning of the cruise and one near the end.  This drift resulted in 				the ship's readings being offset by as much as 1.5 units less than our 				TSG.  I suspect the problem is with the ship's salinity readings 					because of large discontinuities in the plot of salinity vs. time.  I 				used the apparently good readings from June 2 at 1800 to June 7 at 2357 			to compute an average offset of .1116 (Seabird - ship salinity) with a 				standard deviation of 0.03.  I then replaced all the ship's salinities 				before and after this time period with values of Seabird salinity + 				0.1116 in the final data file.

RB200406:	1. Elevated air xCO2 values of greater than 380 ppm were recorded at 				several points during the cruise.  Since these could not be attributed 				to stack gas, it was assumed that they were caused by shore-based 				sources and they have been left in the data set.

2. The ship's salinity sensor readings drifted down relative to our 				Seabird sensor near the equilibrator beginning on June 22 at midnight 				and ending on June 27 at 0922.  This drift resulted in the ship's 				readings being offset by as much as 1.5 units less than our TSG.  I 				suspect the problem is with the ship's salinity readings because of 				large discontinuities in the plot of ship's salinity readings vs. time.  			I used the apparently good readings from the beginning of the cruise to 			the end of June 21 to compute an average offset of .1005 (Seabird - 				ship salinity) with a standard deviation of 0.02.  I then replaced all 				the ship's salinities during this time period with values of Seabird 				salinity + 0.1005 in the final data file.

RB200407A:	1. There is a discontinuity in the salinity read from the ship's TSG on 			July 18 at 0141.  Ship's salinity readings from July 12 at 0152 to July 			18 at 0136 have been corrected using the Seabird TSG near the pCO2 				system.  The new readings are equal to Seabird salinity + 0.206.  This 				equation is based on the last 5-1/2 days of data (ship salinity = 				Seabird salinity + 0.206, SD = 0.084, n = 1868).

2. The sea water flow fell to zero from July 11 at 1436 to 2014.  Water 			values during this time period have been removed.

RB200407B:	1. There are discontinuities in the salinity read from the ship's TSG 				on August 1 at 2227 and on August 5 at 2237.  Ship's salinity readings 				from July 31 at 0232 to August 1 at 2227 and from August 3 at 2015 to 				August 5 at 2337 have been corrected using the Seabird TSG near the 				pCO2 system.  The new readings are equal to Seabird salinity - 0.07.  				This equation is based on all of the data that was not recorded during 				the two bad periods above (ship salinity = Seabird salinity + 0.07, SD 				= 0.086, n = 3791).

2. The sea water flow fell to zero from August 5 at 2319 to August 6 at 			1328 and from August 8 at 1824 to August 9 at 1624.  Water values 				during this time period have been removed.

RB200408:	No problems of note.

RB200409:	No problems of note.

RB200410:	No problems of note.

RB200411	No problems of note.